6 GEODETIC CODES AND PARAMETERS

The 4 main geodetic concepts in this chapter are ellipsoid, datum, projection and grid system.

A geodetic datum includes an ellipsoid as one of its defining components. A grid system includes a datum and a projection among its defining components. The way in which geodetic datum, ellipsoid, grid and projection are inter-related is shown in Figure 6-1.

Figure 6-1 Concept Relationships

The codes identifying ellipsoid, datum, projection and grid are listed in Tables 6-1, 6-2, 6-5 and 6-6.

It should be noted that the grid codes in Table 6-6 are allocated to both grid systems and grid categories. A grid category includes a number of different grids, with variations in geodetic datum and/or zone of application. The most obvious example is Universal Transverse Mercator.

.1 Ellipsoid Codes

The parameters (semimajor axis a and inverse flattening 1/f) are purely to assist ellipsoid identification. The abbreviation "Alt:" is used to denote alternative codes originating from DIGEST 1.2, which are included for backward compatibility.

In some cases, ellipsoids have come into existence as part of a datum definition. As a result, some ellipsoids are known by the same name as the datum, although the codes will differ.

Note the presence of special codes: NO for no ellipsoid, ZY for other known ellipsoid and ZZ for unknown ellipsoid.

Table 6-1 Ellipsoid Codes

| Ellipsoid (followed by parameters a, 1/f) | Ellipsoid Code |
| --- | --- |
| Airy (1830) (6377563.396, 299.3249647) | AA Alt: AAY |
| US - Modified Airy UK - Airy Modified (6377340.189, 299.3249647) | AMAlt: AAM |
| Australian National (1966) (6378160.000, 298.2500000) | AN |
| APL 4.5 (1968) (6378144.000, 298.2300000) | AP |
| Average Terrestrial System 1977 (6378135.000, 298.2570000) | AT |
| Airy (War Office) (6377542.178, 299.3250000) | AW |
| Bessel (Modified) (6377492.018, 299.1528000) | BM |
| Bessel 1841 (Namibia) (6377483.865, 299.1528128) | BN |
| US - Bessel 1841 (Ethiopia, Indonesia, Japan, Korea) UK - Bessel (1841) Revised (6377397.155, 299.1528128) | BR |
| Clarke 1858 (6378235.600, 294.2606768) | CA |
| Clarke 1858 (Modified) (6378293.645, 294.26000000) | CB |
| Clarke 1866 (6378206.400, 294.9786982) | CC Alt: CLK |
| US - Clarke 1880 UK - Clarke 1880 Modified (6378249.145, 293.4650000) | CD Alt: CLJ |
| Clarke 1880 (Cape) (6378249.145, 293.4663077) | CE |
| Clarke 1880 (Palestine) 6378300.782, 293.4663077) | CF |
| Clarke 1880 (IGN) (6378249.200, 293.4660208) | CG |
| Clarke 1880 (Syria) (6378247.842, 293.4663517) | CI |
| Clarke 1880 (Fiji) (6378301.000, 293.4650000) | CJ |
| Clarke 1880 (Unspecified) (-,-) | CL |
| Danish (1876) or Andrae (6377104.430, 300.0000000) | DA |
| Delambre 1810 (6376985.228, 308.6400000) | DB |
| Delambre (Carte de France) (6376985.000, 308.6400000) | DC |
| US - Everest (India 1830) UK - Everest (1830) (6377276.345, 300.8017000) | EA |
| US - Everest (Brunei and E. Malaysia (Sabah and Sarawak)) UK - Everest (Borneo) (6377298.556, 300.8017000) | EB |
| US - Everest (India 1956) UK - Everest (India) (6377301.243, 300.8017000) UK takes 1/f as 300.8017255. | EC |
| US - Everest (W. Malaysia 1969) UK - Everest (Malaya RSO) (6377295.664, 300.8017000) | ED |
| US - Everest (W. Malaysia and Singapore 1948) UK - Everest (Malaya RKT) (6377304.063, 300.8017000) | EE |
| Everest (Pakistan) (6377309.613, 300.8017000) | EF |
| Everest (Unspecified) (-,-) | EV |
| US - Modified Fischer 1960 (South Asia) UK - Fischer 1960 (South Asia) (6378155.000, 298.3000000) | FA |
| Fischer 1968 (6378150.000, 298.3000000) | FC |
| Fischer 1960 (Mercury) (6378166.000, 298.3000000) | FM |
| Germaine (Djibouti) (6378284.000, 294.0000000) | GE |
| Hayford 1909 (6378388.000, 296.9592630) The original version, based on a=6378388, b=6356909. | HA |
| Helmert 1906 (6378200.000, 298.3000000) | HE |
| Hough 1960 (6378270.000, 297.0000000) | HO |
| IAG Best Estimate 1975 (6378140.000, 298.2570000) | IA |
| Indonesian National (1974) (6378160.000, 298.2470000) | ID |
| US - International 1924 UK - International (6378388.000, 297.0000000) | IN Alt: INT |
| Krassovsky (1940) (6378245.000, 298.3000000) | KA Alt: KRA |
| Krayenhoff 1827 (6376950.400, 309.6500000) | KB |
| No ellipsoid | NO |
| NWL-8E (6378145.000, 298.2500000) | NW |
| Plessis Modified (6376523.000, 308.6400000) | PM |
| Plessis Reconstituted (6376523.994, 308.6248070) | PR |
| Geodetic Reference System 1967 (6378160.000, 298.2471674) | RE |
| Geodetic Reference System 1980 (6378137.000, 298.2572221) | RF |
| South American (6378160.000, 298.2500000) | SA |
| Soviet Geodetic System 1985 (6378136.000, 298.2570000) | SG |
| Ellipsoid Junction | SJ |
| Soviet Geodetic System 1990 (6378136.000, 298.2578393) | SN |
| Struve 1860 (6378298.300, 294.7300000) | ST |
| Svanberg (6376797.000, 304.2506000) | SV |
| Walbeck 1819 (Planheft 1942) (6376895.000, 302.7821565) | WA |
| Walbeck 1819 (AMS 1963) (6376896.000, 302.7800000) | WB |
| World Geodetic System 1966 (6378145.000, 298.2500000) | WC |
| World Geodetic System 1972 (6378135.000, 298.2600000) | WD Alt: WGC |
| World Geodetic System 1984 (6378137.000, 298.2572236) | WE Alt: WGE |
| World Geodetic System (Unspecified) (-,-) | WF |
| US - War Office 1924 (McCaw) UK - War Office 1924 (6378300.000, 296.0000000) | WO |
| World Geodetic System 1960 (6378165.000, 298.3000000) | WS |
| Other Known Ellipsoid | ZY |
| Unknown Ellipsoid | ZZ |

6.2 Datum Codes

Table 6-2 provides the allowable datums and their codes for the Geodetic Datum fields. Details of transformations between most of the datums and WGS84 can be found in DMA Technical Report 8350.2.

In some cases a geodetic datum with a 3-letter code is followed by 4-letter codes referring to the same datum but specifying particular regions. See, for example, codes AINA and AINB which follow AIN. The 4-letter codes are not different datums, but "regional" solutions to the datum. Regional solutions represent regional variations in the datums relationship with WGS 1984 (arising from regional distortions in the datum). Use of the 4-letter code is recommended when there is a need to identify that relationship.

Unless indicated otherwise at the end of the datum name, the Zero Meridian is always Greenwich. Datums with a zero meridian other than Greenwich have "1" as a 4th character in the datum code.

To assist the process of matching ellipsoids to datums, ellipsoid codes are shown in the final column.

The abbreviation "Alt:" is used to denote alternative codes originating from DIGEST 1.2, which are included for backward compatibility.

Note the presence of special codes:

Geodetic Datums (Table 6-2): UND for undetermined datum and ZYX for other known datum.

Sounding Datums (Table 6-4): ZYX for other known sounding datum and ZZZ for unknown.

Table 6-2 Geodetic Datum Codes

| Geodetic Datums Each Horizontal Datum below can also be used as a Vertical Datum (but only in the case where the ellipsoid is the surface from which elevation is measured) | Datum Code | Ellipsoid Code |
| --- | --- | --- |
| Adindan | ADI | CD |
| Adindan (Ethiopia) | ADIA | CD |
| Adindan (Sudan) | ADIB | CD |
| Adindan (Mali) | ADIC | CD |
| Adindan (Senegal) | ADID | CD |
| Adindan (Burkina Faso) | ADIE | CD |
| Adindan (Cameroon) | ADIF | CD |
| Adindan (Mean value: Ethiopia and Sudan) | ADIM | CD |
| Afgooye (Somalia) | AFG | KA |
| Antigua Island Astro 1943 | AIA | CD |
| Ain el Abd 1970 | AIN | IN |
| Ain el Abd 1970 (Bahrain Island) | AINA | IN |
| Ain el Abd 1970 (Saudi Arabia) | AINB | IN |
| American Samoa Datum 1962 | AMA | CC |
| Amersfoort 1885/1903 (Netherlands) | AME | BR |
| Anna 1 Astro 1965 (Cocos Islands) | ANO | AN |
| Approximate Luzon Datum (Philippines) | APL | CC |
| Arc 1950 | ARF | CD |
| Arc 1950 (Botswana) | ARFA | CD |
| Arc 1950 (Lesotho) | ARFB | CD |
| Arc 1950 (Malawi) | ARFC | CD |
| Arc 1950 (Swaziland) | ARFD | CD |
| Arc 1950 (Zaire) | ARFE | CD |
| Arc 1950 (Zambia) | ARFF | CD |
| Arc 1950 (Zimbabwe) | ARFG | CD |
| Arc 1950 (Burundi) | ARFH | CD |
| Arc 1950 (Mean value: Botswana, Lesotho, Malawi, Swaziland, Zaire, Zambia, and Zimbabwe) | ARFM | CD |
| Arc 1960 | ARS | CD |
| Arc 1960 (Kenya) | ARSA | CD |
| Arc 1960 (Tanzania) | ARSB | CD |
| Arc 1960 (Mean value: Kenya, Tanzania) | ARSM | CD |
| Arc 1935 (Africa) | ART | CD |
| Ascension Island 1958 (Ascension Island) | ASC | IN |
| Montserrat Island Astro 1958 | ASM | CD |
| Astro Station 1952 (Marcus Island) | ASQ | IN |
| Astro Beacon "E" (Iwo Jima Island) | ATF | IN |
| Average Terrestrial System 1977, New Brunswick | ATX | AT |
| Australian Geod. 1966 (Australia and Tasmania Is.) | AUA | AN |
| Australian Geod. 1984 (Australia and Tasmania Is.) | AUG | AN |
| Djakarta (Batavia) (Sumatra Island, Indonesia) | BAT | BN |
| Djakarta (Batavia) (Sumatra Island, Indonesia) with Zero Meridian Djakarta | BAT1 | BN |
| Bekaa Base South End (Lebanon) | BEK | CG |
| Belgium 1950 System (Lommel Signal, Belgium) | BEL | IN |
| Bermuda 1957 (Bermuda Islands) | BER | CC |
| Bissau (Guinea-Bissau) | BID | IN |
| Modified BJZ54 (China) | BJM | KA |
| BJZ54 (A954 Beijing Coordinates) (China) | BJZ | KA |
| Bogota Observatory (Colombia) | BOO | IN |
| Bogota Observatory (Colombia) with Zero Meridian Bogota | BOO1 | IN |
| Bern 1898 (Switzerland) | BRE | BR |
| Bern 1898 (Switzerland) with Zero Meridian Bern | BRE1 | BR |
| Bukit Rimpah (Bangka & Belitung Islands, Indonesia) | BUR | BR |
| Cape Canaveral (Mean value: Florida and Bahama Islands) | CAC | CC |
| Campo Inchauspe (Argentina) | CAI | IN |
| Camacupa Base SW End (Campo De Aviacao, Angola) | CAM | CD |
| Canton Astro 1966 (Phoenix Islands) | CAO | IN |
| Cape (South Africa) | CAP | CE |
| Camp Area Astro (Camp McMurdo Area, Antarctica) | CAZ | IN |
| S-JTSK, Czechoslavakia (prior to 1 Jan 1993) | CCD | BN |
| Carthage (Tunisia) | CGE | CG |
| Compensation Géodétique du Québec 1977 | CGX | CC |
| Chatham 1971 (Chatham Island, New Zealand) | CHI | IN |
| Chua Astro (Paraguay) | CHU |  |
| Corrego Alegre (Brazil) | COA | IN |
| Conakry Pyramid of the Service Geographique (Guinea) | COV | CG |
| Guyana CSG67 | CSG |  |
| Dabola (Guinea) | DAL | CD |
| DCS-3 Lighthouse, Saint Lucia, Lesser Antilles | DCS | CD |
| Deception Island, Antarctica | DID | CD |
| GUX 1 Astro (Guadacanal Island) | DOB | IN |
| Dominica Astro M-12, Dominica, Lesser Antilles | DOM |  |
| Easter Island 1967 (Easter Island) | EAS | IN |
| Wake-Eniwetok 1960 (Marshall Islands) | ENW | HO |
| European 1950 | EUR | IN |
| European 1950 (Western Europe: Austria, Denmark, France, Federal Republic of Germany, Netherlands, and Switzerland) | EURA | IN |
| European 1950 (Greece) | EURB | IN |
| European 1950 (Norway and Finland) | EURC | IN |
| European 1950 (Portugal and Spain) | EURD | IN |
| European 1950 (Cyprus) | EURE | IN |
| European 1950 (Egypt) | EURF | IN |
| European 1950 (England, Channel Islands, Scotland, and Shetland Islands) | EURG | IN |
| European 1950 (Iran) | EURH | IN |
| European 1950 (Sardinia) | EURI | IN |
| European 1950 (Sicily) | EURJ | IN |
| European 1950 (England, Channel Islands, Ireland, Northern Ireland, Scotland, Shetland Islands, and Wales) | EURK | IN |
| European 1950 (Malta) | EURL | IN |
| European 1950 (Mean value: Austria, Belgium, Denmark, Finland, France, Federal Republic of Germany, Gibraltar, Greece, Italy, Luxembourg, Netherlands, Norway, Portugal, Spain, Sweden, & Switzerland) | EURM | IN |
| European 1950 (Iraq, Israel, Jordan, Kuwait, Lebanon, Saudi Arabia, and Syria) | EURS | IN |
| European 1950 (Tunisia) | EURT | IN |
| European 1979 (Mean value: Austria, Finland, Netherlands, Norway, Spain, Sweden, and Switzerland) | EUS | IN |
| European Terrestrial Reference System 1989 (ETRS89) | EUT | RF |
| Oman (Oman) | FAH | CD |
| Observatorio Meteorologico 1939 (Corvo and Flores Islands, Azores) | FLO | IN |
| Fort Thomas 1955 (Nevis, St Kitts, Leeward Islands) | FOT | CD |
| Gan 1970 (Addu Atoll, Republic of Maldives) | GAA | IN |
| Gandajika Base (Zaire) | GAN | IN |
| Geocentric Datum of Australia (GDA) | GDS | RF |
| GDZ80 (China) | GDZ | IA |
| Geodetic Datum 1949 (New Zealand) | GEO | IN |
| DOS 1968 (Gizo Island, New Georgia Islands) | GIZ | IN |
| Graciosa Base SW (Faial, Graciosa, Pico, Sao Jorge, and Terceira Island, Azores) | GRA | IN |
| Greek Datum, Greece | GRK | BR |
| Greek Geodetic Reference System 1987 (GGRS 87) | GRX | RF |
| Gunong Segara (Kalimantan Island, Indonesia) | GSE | BR |
| Gunong Serindung | GSF | BR |
| Guam 1963 | GUA | CC |
| Herat North (Afganistan) | HEN | IN |
| Hermannskogel | HER | BR |
| Provisional South Chilean 1963 (or Hito XVIII 1963) (S. Chile, 53°S) | HIT | IN |
| Hjörsey 1955 (Iceland) | HJO | IN |
| Hong Kong 1963 (Hong Kong) | HKD | IN |
| Hong Kong 1929 | HKO | CA |
| Hu-Tzu-Shan | HTN | IN |
| Hungarian 1972 | HUY | RE |
| Bellevue (IGN) (Efate and Erromango Islands) | IBE | IN |
| Indonesian 1974 | IDN | ID |
| Indian | IND |  |
| Indian (Thailand and Vietnam) | INDA |  |
| Indian (Bangladesh) | INDB | EA |
| Indian (India and Nepal) | INDI | EC |
| Indian (Pakistan) | INDP | EF |
| Indian (1954) | INF | EA |
| Indian 1954 (Thailand) | INFA | EA |
| Indian 1960 | ING | EA |
| Indian 1960 (Vietnam: near 16°N) | INGA | EA |
| Indian 1960 (Con Son Island (Vietnam)) | INGB | EA |
| Indian 1975 | INH | EA |
| Indian 1975 (Thailand) | INHA | EA |
| Ireland 1965 (Ireland and Northern Ireland) | IRL | AM |
| ISTS 061 Astro 1968 (South Georgia Islands) | ISG | IN |
| ISTS 073 Astro 1969 (Diego Garcia) | IST | IN |
| Johnston Island 1961 (Johnston Island) | JOH | IN |
| Kalianpur (India) | KAB | EC |
| Kandawala (Sri Lanka) | KAN | EA |
| Kertau 1948 (or Revised Kertau) (West Malaysia and Singapore) | KEA | EE |
| KCS 2, Sierra Leone | KCS | WO |
| Kerguelen Island 1949 (Kerguelen Island) | KEG | IN |
| Korean Geodetic System 1995 (South Korea) | KGS | RF |
| KKJ (or Kartastokoordinaattijarjestelma), Finland | KKX | IN |
| Kusaie Astro 1951 | KUS | IN |
| Kuwait Oil Company (K28) | KUW | CD |
| L.C. 5 Astro 1961 (Cayman Brac Island) | LCF | CC |
| Leigon (Ghana) | LEH | CG |
| Liberia 1964 (Liberia) | LIB | CD |
| Lisbon (Castelo di São Jorge), Portugal | LIS |  |
| Local Astro. | LOC |  |
| Loma Quintana (Venezuela) | LOM | IN |
| Luzon | LUZ | CC |
| Luzon (Philipines except Mindanao Island) | LUZA | CC |
| Luzon (Mindanao Island) | LUZB | CC |
| Marco Astro (Salvage Islands) | MAA | IN |
| Martinique Fort-Desaix | MAR | IN |
| Massawa (Eritrea, Ethiopia) | MAS | BR |
| Manokwari (West Irian) | MAW |  |
| Mayotte Combani | MCX | IN |
| Mount Dillon, Tobago | MDT | CB |
| Merchich (Morocco) | MER | CG |
| Midway Astro 1961 (Midway Island) | MID | IN |
| Mahe 1971 (Mahe Island) | MIK | CD |
| Minna | MIN | CD |
| Minna (Cameroon) | MINA | CD |
| Minna (Nigeria) | MINB | CD |
| Rome 1940 (or Monte Mario 1940), Italy | MOD | IN |
| Rome 1940 (or Monte Mario 1940), Italy, with Zero Meridian Rome | MOD1 | IN |
| Montjong Lowe | MOL | BR |
| M'Poraloko (Gabon) | MPO | CD |
| Viti Levu 1916 (Viti Levu Island, Fiji Islands) | MVS | CD |
| Nahrwan | NAH | CD |
| Nahrwan (Masirah Island, Oman) | NAHA | CD |
| Nahrwan (United Arab Emirates) | NAHB | CD |
| Nahrwan (Saudi Arabia) | NAHC | CD |
| Naparima (BWI, Trinidad and Tobago) | NAP | IN |
| North American 1983 | NAR | RF |
| North American 1983 (Alaska, excluding Aleutian Islands) | NARA | RF |
| North American 1983 (Canada) | NARB | RF |
| North American 1983 (CONUS) | NARC | RF |
| North American 1983 (Mexico and Central America)) | NARD | RF |
| North American 1983 (Aleutian Islands) | NARE | RF |
| North American 1983 (Hawaii) | NARH | RF |
| North American 1927 | NAS | CC |
| North American 1927 (Eastern US) | NASA | CC |
| North American 1927 (Western US) | NASB | CC |
| North American 1927 (Mean value: CONUS) | NASC | CC |
| North American 1927 (Alaska) | NASD | CC |
| North American 1927 (Mean value: Canada) | NASE | CC |
| North American 1927 (Alberta and British Columbia) | NASF | CC |
| North American 1927 (Newfoundland, New Brunswick, Nova Scotia and Quebec) | NASG | CC |
| North American 1927 (Manitoba and Ontario) | NASH | CC |
| North American 1927 (Northwest Territories and Saskatchewan) | NASI | CC |
| North American 1927 (Yukon) | NASJ | CC |
| North American 1927 (Mexico) | NASL | CC |
| North American 1927 (Central America - Belize, Costa Rica, El Salvador, Guatemala, Honduras, and Nicaragua) | NASN | CC |
| North American 1927 (Canal Zone) | NASO | CC |
| North American 1927 (Caribbean, Barbados, Caicos Islands, Cuba, Dominican Republic, Grand Cayman, Jamaica, Leeward Islands, and Turks Islands) | NASP | CC |
| North American 1927 (Bahamas, except San Salvador Island) | NASQ | CC |
| North American 1927 (San Salvador Island) | NASR | CC |
| North American 1927 (Cuba) | NAST | CC |
| North American 1927 (Hayes Peninsula, Greenland) | NASU | CC |
| North American 1927 (Aleutian Islands East of 180°W) | NASV | CC |
| North American 1927 (Aleutian Islands West of 180°W) | NASW | CC |
| Revised Nahrwan | NAX | CD |
| New French or Nouvelle Triangulation Française (NTF) with Zero Meridian Paris | NFR1 Alt: FDA | CG |
| North Sahara 1959 | NSD | CD |
| Ocotopeque, Guatemala | OCO |  |
| Belgium 1972 (Observatoire d'Uccle) | ODU | IN |
| Old Egyptian (Egypt) | OEG | HE |
| Ordnance Survey of Great Britain 1936 | OGB | AA |
| Ordnance Survey G.B. 1936 (England) | OGBA | AA |
| Ordnance Survey G.B. 1936 (England, Isle of Man, and Wales) | OGBB | AA |
| Ordnance Survey G.B. 1936 (Scotland and Shetland Islands) | OGBC | AA |
| Ordnance Survey G.B. 1936 (Wales) | OGBD | AA |
| Ordnance Survey G.B. 1936 (Mean value: England, Isle of Man, Scotland, Shetland, and Wales) | OGBM | AA |
| Old Hawaiian | OHA | CC |
| Old Hawaiian (Hawaii) | OHAA | CC |
| Old Hawaiian (Kauai) | OHAB | CC |
| Old Hawaiian (Maui) | OHAC | CC |
| Old Hawaiian (Oahu) | OHAD | CC |
| Old Hawaiian (Mean value) | OHAM | CC |
| Oslo Observatory (Old), Norway | OSL | BR |
| Padang Base West End (Sumatra, Indonesia) | PAD | BR |
| Padang Base West End (Sumatra, Indonesia) with Zero Meridian Djakarta | PAD1 | BR |
| Palestine 1928 (Israel, Jordan) | PAL | CF |
| Potsdam or Helmertturm (Germany) | PDM | IN |
| Ayabelle Lighthouse (Djibouti) | PHA | CD |
| Pitcairn Astro 1967 (Pitcairn Island) | PIT | IN |
| Pico de las Nieves (Canary Islands) | PLN | IN |
| SE Base (Porto Santo) (Porto Santo & Madeira Islands) | POS | IN |
| Provisional South American 1956 | PRP | IN |
| Prov. S. American 1956 (Bolivia) | PRPA | IN |
| Prov. S. American 1956 (Northern Chile near 19°S) | PRPB | IN |
| Prov. S. American 1956 (Southern Chile near 43°S) | PRPC | IN |
| Prov. S. American 1956 (Columbia) | PRPD | IN |
| Prov. S. American 1956 (Ecuador) | PRPE | IN |
| Prov. S. American 1956 (Guyana) | PRPF | IN |
| Prov. S. American 1956 (Peru) | PRPG | IN |
| Prov. S. American 1956 (Venezuela) | PRPH | IN |
| Prov. S. American 1956 (Mean value: Bolivia, Chile, Colombia, Ecuador, Guyana, Peru, & Venezuela) | PRPM | IN |
| Point 58 Mean Solution (Burkina Faso and Niger) | PTB | CD |
| Pointe Noire 1948 | PTN | CD |
| Pulkovo 1942 (Russia) | PUK | KA |
| Puerto Rico (Puerto Rico and Virgin Islands) | PUR | CC |
| Qatar National (Qatar) | QAT | IN |
| Qornoq (South Greenland) | QUO | IN |
| Rauenberg (Berlin, Germany) | RAU | BR |
| Reconnaissance Triangulation, Morocco | REC | CG |
| Reunion 1947 | REU | IN |
| RT90, Stockholm, Sweden | RTS | BR |
| Santo (DOS) 1965 (Espirito Santo Island) | SAE | IN |
| South African (South Africa) | SAF | CD |
| Sainte Anne I 1984 (Guadeloupe) | SAG |  |
| South American 1969 | SAN | SA |
| South American 1969 (Argentina) | SANA | SA |
| South American 1969 (Bolivia) | SANB | SA |
| South American 1969 (Brazil) | SANC | SA |
| South American 1969 (Chile) | SAND | SA |
| South American 1969 (Columbia) | SANE | SA |
| South American 1969 (Ecuador) | SANF | SA |
| South American 1969 (Guyana) | SANG | SA |
| South American 1969 (Paraguay) | SANH | SA |
| South American 1969 (Peru) | SANI | SA |
| South American 1969 (Baltra, Galapagos Islands) | SANJ | SA |
| South American 1969 (Trinidad and Tobago) | SANK | SA |
| South American 1969 (Venezuela) | SANL | SA |
| South American 1969 (Mean value: Argentina, Bolivia, Brazil, Chile, Columbia, Ecuador, Guyana, Paraguay, Peru, Trinidad and Tobago, and Venezuela) | SANM | SA |
| Sao Braz (Sao Miguel, Santa Maria Islands, Azores) | SAO | IN |
| Sapper Hill 1943 (East Falkland Islands) | SAP | IN |
| Schwarzeck (Namibia) | SCK | BN |
| Soviet Geodetic System 1985 | SGA | SG |
| Soviet Geodetic System 1990 | SGB | SG |
| Selvagem Grande 1938 (Salvage Islands) | SGM | IN |
| Astro Dos 71/4 (St. Helena Island) | SHB | IN |
| Sierra Leone 1960 | SIB | CD |
| South Asia (Southeast Asia, Singapore) | SOA | FA |
| S-42 (Pulkovo 1942) | SPK | KA |
| St. Pierre et Miquelon 1950 | SPX | CC |
| Stockholm 1938 (Sweden) | STO | BR |
| Sydney Observatory, New South Wales, Australia | SYO | CB |
| Tananarive Observatory 1925 | TAN | IN |
| Tananarive Observatory 1925, with Zero Meridian Paris | TAN1 | IN |
| Tristan Astro 1968 (Tristan da Cunha) | TDC | IN |
| Timbalai 1948 (Brunei and East Malaysia - Sarawak and Sabah) | TIL | EB |
| Timbali 1968 | TIN |  |
| Tokyo | TOY | BR |
| Tokyo (Japan) | TOYA | BR |
| Tokyo (Korea) | TOYB | BR |
| Tokyo (Okinawa) | TOYC | BR |
| Tokyo (Mean value: Japan, Korea, and Okinawa) | TOYM | BR |
| Trinidad 1903 | TRI | CA |
| Astro Tern Is. 1961 (Tern Island, Hawaii) | TRN | IN |
| Undetermined or Unknown (see Note 1) | UND |  |
| Voirol 1875 | VOI | CG |
| Voirol 1875 with Zero Meridian Paris | VOI1 | CG |
| Voirol 1960, Algeria | VOR | CG |
| Voirol 1960, Algeria, with Zero Meridian Paris | VOR1 | CG |
| Wake Island Astro 1952 | WAK |  |
| World Geodetic System 1960 | WGA | WS |
| World Geodetic System 1966 | WGB | WC |
| World Geodetic System 1972 | WGC | WD |
| World Geodetic System 1984 | WGE | WE |
| Yacare (Uruguay) | YAC | IN |
| Zanderij (Surinam) | ZAN | IN |
| Other Known Datum | ZYX |  |

Note 1. In the case of an unknown or unspecified datum, it is potentially dangerous to assume a default of WGS 84. Unless the application is small-scale mapping and the zero meridian is known to be Greenwich, such an assumption could cause significant positioning errors.

Table 6-3 Codes for Vertical Datums

| Vertical Datum Reference | Code |
| --- | --- |
| Geodetic (see Note 1) | GEOD |
| Mean Sea Level (see Note 2) with identification Example: Mean Sea Level Singapore | MSL |

Note 1. In the geodetic case, all elevations in the dataset are referenced to the ellipsoid of the specified geodetic datum, which means they are ellipsoidal heights.

Note 2. In the mean-sea-level case, all elevations in the dataset are referenced to a particular determination of mean sea level.

Table 6-4 Codes for Sounding Datums

| Sounding Datum | Code |
| --- | --- |
| Approximate Lowest Astronomical Tide | ALAT |
| Approximate Mean Low Water Springs | AMLS |
| Approximate Mean Low Water Tide | AMLT |
| Approximate Mean Low Water | AMLW |
| Approximate Mean Sea Level | AMSL |
| Chart Datum (Unspecified) | CD |
| Equinoctial Spring Low Water | ESLW |
| Highest Astronomical Tide | HAT |
| Higher High Water Large Tide | HHLT |
| Highest Normal High Water | HNHR |
| Higher High Water | HRHW |
| Highest High Water | HTHW |
| High Water | HW |
| High Water Springs | HWS |
| International Great Lakes Datum 1985 | IGLD |
| Indian Spring High Water | ISHW |
| Indian Spring Low Water | ISLW |
| Lowest Astronomical Tide | LAT |
| Local Datum (arbitrary datum defined by local harbour authority) | LD |
| Lower Low Water Large Tide | LLLT |
| Lowest Low Water Springs | LLWS |
| Lower Low Water | LRLW |
| Lowest Low Water | LTLW |
| Low Water | LW |
| Low Water Springs | LWS |
| Mean Higher High Water | MHHW |
| Mean Higher Water | MHRW |
| Mean High Water | MHW |
| Mean High Water Neaps | MHWN |
| Mean High Water Springs | MHWS |
| Mean Lower Low Water Springs | MLLS |
| Mean Lower Low Water | MLLW |
| Mean Low Water | MLW |
| Mean Low Water Neaps | MLWN |
| Mean Low Water Springs | MLWS |
| Mean Sea | MSL |
| Mean Tide Level | MTL |
| Nearly Lowest Low Water | NLLW |
| Neap Tide | NT |
| Spring Tide | ST |
| VALUE INTENTIONALLY LEFT BLANK | VILB |
| Other Known Sounding Datum | ZYX |
| Unknown | ZZZ |

6.3 Projection Codes and Parameters

Table 6-5 provides the allowable projections and their codes and parameters for the Dataset Map Projection Group. These codes and parameters are necessary for conversion of geographic coordinates to/from grid coordinates (as used on a map). Also needed are Easting False Origin and Northing False Origin, as well as the geodetic datum.

The abbreviation "Alt:" is used to denote alternative codes originating from DIGEST 1.2, which are included for backward compatibility.

Note the presence of a special code ZY for other known projection.

Table 6-5 Projection Codes and Parameters

| Projection | Proj'n Code | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 |  |  |  |
| Albers Equal-Area Conic | AC | Longitude of Origin | Std. Parallel Nearer to Equator | Std. Parallel Farther from Equator | Latitude of Origin (see Note 5) |  |  |  |
| (Lambert) Azimuthal Equal-Area | AK | Longitude of Proj. Origin | Latitude of Proj. Origin | - | - |  |  |  |
| Azimuthal Equidistant | AL | Longitude of Proj. Origin | Latitude of Proj. Origin | - | - |  |  |  |
| Bonne | BF | Longitude of Proj. Origin | Latitude of Proj. Origin | Scale Factor at Proj. Origin | - |  |  |  |
| Equirectangular (La Carte Parallélogramatique) | CC | Longitude of Central Meridian | Latitude of True Scale | - | - |  |  |  |
| Equidistant Conic with 1 Standard Parallel | CP | Longitude of Central Meridian | Latitude of Proj. Origin | Latitude of Standard Parallel | - |  |  |  |
| Cassini-Soldner | CS | Longitude of Proj. Origin | Latitude of Proj. Origin | - | - |  |  |  |
| Gnomonic | GN | Longitude of Proj. Origin | Latitude of Proj. Origin | - | - |  |  |  |
| Hotine Oblique Mercator based on 2 Points | HX | Scale Factor at Proj. Origin | Latitude of Proj. Origin | Longitude of 1st Point defining Central Line | Latitude of 1st Point defining Central Line |  |  |  |
| (Note the 5th and 6th Parameters shown right.) |  | Longitude of 2nd Point defining Central Line | Latitude of 2nd Point defining Central Line | - | - |  |  |  |
| Equidistant Conic with 2 Standard Parallels | KA | Longitude of Central Meridian | Latitude of Origin (see Note 5) | Latitude of Standard Parallel Nearer to Equator | Latitude of Standard Parallel Farther from Equator |  |  |  |
| Laborde | LA | Longitude of Proj. Origin | Latitude of Proj. Origin | Scale Factor at Proj. Origin | - |  |  |  |
| Lambert Conformal Conic (see Note 1) | LE | Longitude of Origin | Std. Parallel Nearer to Equator | Std Parallel Farther from Equator | Latitude of Origin (see Note 5) |  |  |  |
| Lambert Equal-Area Meridional | LJ | Longitude of Central Meridian | Latitude of Proj. Origin | - | - |  |  |  |
| Mercator | MC | Longitude of Central Meridian | Latitude of True Scale | - (see Note 6) | - |  |  |  |
| Miller Cylindrical | MH | Longitude of Central Meridian | Radius of Sphere (see Note 2) | - | - |  |  |  |
| French Lambert | MJ | Longitude of Proj. Origin | Latitude of Proj. Origin | Scale Factor at Proj. Origin | - |  |  |  |
| New Zealand Map Grid | NT | Longitude of Proj. Origin | Latitude of Proj. Origin | - | - |  |  |  |
| Oblique Mercator | OC | Longitude of Reference Point on Great Circle | Latitude of Reference Point on Great Circle | Azimuth of Great Circle at Reference Point | - |  |  |  |
| Orthographic | OD | Longitude of Proj. Origin | Latitude of Proj. Origin | - | - |  |  |  |
| Polar Stereographic | PG | Central Meridian (Longitude straight down from Pole on map) | Latitude of True Scale | - | - |  |  |  |
| Polyconic | PH | Longitude of Central Meridian | Latitude of Proj. Origin | - | - |  |  |  |
| Relative Coordinates | RC | X-Scale Factor | Y-Scale Factor | - | - |  |  |  |
| Hotine Oblique Mercator (Rectified Skew Orthomorphic) | RS Alt:RB | Longitude of Proj. Origin | Latitude of Proj. Origin | Azimuth East of North for Central Line (Skew X-Axis) at Proj. Origin | Scale Factor at Proj. Origin |  |  |  |
| Robinson | RX | Longitude of Central Meridian | Radius of Sphere (see Note 2) | - | - |  |  |  |
| Sinusoidal | SA | Longitude of Central Meridian | Radius of Sphere (see Note 2) | - | - |  |  |  |
| Oblique Stereographic | SD | Longitude of Origin | Latitude of Origin | Scale factor at Origin | - |  |  |  |
| Space Oblique Mercator | SX | Application Code (see Note 3) | Vehicle Number (see Note 4) | Orbital Path Number (see Note 4) |  |  |  |  |
| Transverse Mercator | TC | Longitude of Central Meridian | Central Scale Factor | Latitude of Origin (see Note 5) | - |  |  |  |
| Van der Grinten | VA | Longitude of Central Meridian | Radius of Sphere (see Note 2) | - | - |  |  |  |
| General Vertical Near-Side Perspective | VX | Longitude of Proj. Origin | Latitude of Proj. Origin | Height of Perspective Point above Surface (in metres) | - |  |  |  |
| Other Known Projection | ZY | - | - | - | - |  |  |  |

Note 1. The parameters of the Lambert Conformal Conic projection are based on the version derived from 2 Standard Parallels. Where the projection is derived from a single standard parallel with a scale factor, data producers need to compute the equivalent parameters for the 2-standard-parallel case.

Note 2. This radius can be omitted if the chosen sphere has the same surface area as the chosen ellipsoid. The radius R which has that property may be derived from the ellipsoid parameters as follows:

Compute e2 and e from e2 = 2*f - f2.

Qp = 1 - ((1-e2)/(2*e))*Ln((1-e)/(1+e)).

R = a*Sqrt(Qp/2).

Note 3. Application Code:

1 = "Landsat, USGS equations".

2 = "Landsat, EOSAT equations".

(Other values to be added as and when required.)

Note 4. These parameters combined with the Application Code determine the mathematical parameters used in the projection.

Note 5. The Origin included here is the point where Easting False Origin and Northing False Origin are applied, rather than the Projection Origin.

Note 6. For this projection, the Latitude of Origin (in the sense of where to apply Northing False Origin) is the Equator.

6.4 Grid Codes

Table 6-6 provides the allowable grids and their codes for the Grid System field.

To assist the process of matching datums and projections to grids, datum codes and projection codes are shown in the last 2 columns.

It should be noted that some of the entries are grid categories, that is to say there is more than one possible grid. This can be due to more than one possible datum or more than one possible zone, or indeed both. In a small number of cases, a grid category covers zones which use different projections. Grid categories are marked with an asterisk (*).

In the context of a DIGEST dataset, the possible ambiguity of a grid category is resolved when the datum, projection and the values of the projection parameters are specified. Zone number may also be specified to improve identification.

Note the presence of special code MS for other known grid.

Table 6-6 Grid Codes

| Grid Description | Grid Code | Datum Code | Proj'n Code |
| --- | --- | --- | --- |
| Aden Zone | AD |  | LE |
| Afghanistan Gauss-Krüger Grid | AF |  | TC |
| Air Defense Grid | AG |  |  |
| Air Support Grid | AI |  |  |
| Alabama Coordinate System * (see Note 2) | AJ |  | TC |
| Alaska Coordinate System * (see Notes 1 and 2) | AK |  |  |
| Algeria Zone * | AL |  | MJ |
| Albania Bonne Grid | AM |  | BF |
| Alpha-Numeric (Atlas) Grid | AN |  |  |
| Arbitrary Grid | AO |  |  |
| American Samoa Coordinate System * (see Note 2) | AP |  | LE |
| Argentine Gauss-Krüger Conformal Grid * | AQ |  | TC |
| Artillery Referencing System | AR |  |  |
| Arizona Coordinate System * (see Note 2) | AS |  | TC |
| Map Grid of Australia 1994 (MGA94) * | AT | GDS | TC |
| Australia Belt * | AU |  | TC |
| Arkansas Coordinate System * (see Note 2) | AV |  | LE |
| Australian Map Grid * | AW |  | TC |
| Azores Gauss Conformal Grid | AX | LOC | TC |
| Azores Zone | AZ | LOC | LE |
| Baku 1927 Coordinate System | BA |  |  |
| Bavaria Soldner Coordinate System | BB |  |  |
| Belgium Lambert Grid * | BC |  |  |
| Baltic Region Transverse Mercator Grid | BD | EUT | TC |
| Belgium Bonne Grid | BE |  | BF |
| Brazil Gauss Conformal Grid * | BF |  | TC |
| Soldner-Berlin (Müggelberg) Grid | BL | RAU |  |
| Borneo Rectified Skew Orthomorphic Grid * | BO |  | RS |
| British West Indies Grid * | BW |  | TC |
| California Coordinate System * (see Note 2) | CB |  | LE |
| Canada British Modified Grid | CD |  |  |
| Ceylon Belt (Transverse Mercator) | CE | IND | TC |
| Canary Islands (Spanish Lambert Grid) | CF |  |  |
| Chile Gauss Conformal Grid * | CG |  | TC |
| China Belt * | CH |  | TC |
| Canary Islands Zone | CI |  | LE |
| China Lambert Zone | CJ |  | LE |
| Colorado Coordinate Zone * (see Note 2) | CK |  | LE |
| Connecticut Coordinate System * (see Note 2) | CM |  | LE |
| Caspian Zone | CN |  | LE |
| Costa Rica Lambert Grid | CO | OCO | LE |
| Crimea Grid | CQ |  | LE |
| Crete Zone | CR |  | LE |
| Cuba Lambert Grid * | CT | NAS | LE |
| Caucasus Zone | CU | NAH | LE |
| Cape Verde Islands Zone | CV |  | LE |
| British Cassini Grid * | CW | OGB | CS |
| Czechoslovak Uniform Cadastral Coordinate System | CX |  |  |
| Cyprus Grid * | CY |  | CS |
| Czechoslovak Military Grid | CZ | HER | OG |
| Danube Zone | DA | GRK | LE |
| Dahomey Belt | DB |  |  |
| Denmark General Staff Grid | DC |  |  |
| Delaware Coordinate System * (see Note 2) | DD |  | TC |
| Dominican Lambert Grid | DE |  | LE |
| Denmark Geodetic Institute System 1934 | DJ |  | BE |
| Cape Verde Peninsula Grid | DK |  |  |
| East Africa Belt * | EA |  | TC |
| English Belt | EB |  | TC |
| Egypt Gauss Conformal Grid * | ED |  | TC |
| El Salvador Lambert Grid | EE |  | LE |
| Estonian Grid | EF |  |  |
| Estonial lambert Conformal Grid | EL | EUT | LE |
| Hungarian Unified National Mapping System (EOTR) | EO | HUY | TC |
| Egypt Purple Belt | EP |  | TC |
| Egypt Red Belt * | ER |  | TC |
| Egypt 35 Degree Belt | ET | OEG |  |
| Fernando Poo Gauss Grid | FA |  |  |
| Fiji Grid | FB |  |  |
| Florida Coordinate System * (see Notes 1 and 2) | FC |  |  |
| French Bonne Grid | FD |  | BF |
| French Guiana Gauss Grid | FE |  | TC |
| French Somaliland Gauss-Laborde Grid | FF |  |  |
| French Indochina Grid | FI |  |  |
| Franz Josef Land Zone | FJ |  | LE |
| French Lambert Grid * | FL |  | MJ |
| Formosa (Taiwan) Gauss-Schreiber Coordinate System | FO |  |  |
| French Equatorial Africa Grid | FS |  |  |
| Gabon Belt * | GA |  | TC |
| Gauss-Boaga Grid (Transverse Mercator) | GB | EUR | TC |
| Gabon Gauss Conformal Grid | GC |  | TC |
| Geographic Reference System (GEOREF) * | GE |  |  |
| Guadeloupe Gauss-Laborde Grid | GF |  |  |
| Colombia Gauss Conformal Grid | GG | BOO | TC |
| Sweden Gauss-Hannover Grid | GH |  | TC |
| Georgia Coordinate System * (see Note 2) | GI |  | TC |
| Gauss-Krüger Grid (Transverse Mercator) * | GK |  | TC |
| Greece Azimuthal Grid | GL |  |  |
| German Army Grid (DHG) * | GN |  | TC |
| Ghana National Grid | GO |  | TC |
| Greece Bonne Grid | GP |  | BF |
| Greece Conical Mecklenburg Coordinates | GQ |  | LE |
| Greece Conical Mecklenburg Coordinate (New Numbering) | GR |  | LE |
| Greenland Lambert Grid | GT | NAS | LE |
| Guinea Zone | GU |  | LE |
| Guam Coordinate System | GV |  |  |
| Guatemala Lambert Grid | GW |  | LE |
| Guyana Transverse Mercator Grid | GY | LOC | TC |
| Haiti Lambert Grid | HB |  | LE |
| Hawaii Coordinate System * (see Note 2) | HC |  | TC |
| Hawaii Grid | HD |  |  |
| Honduras Lambert Grid | HE |  | LE |
| Hong Kong New System Cassini Grid | HF | HKO | CS |
| Hungary Stereographic Grid | HG | LOC |  |
| Hong Kong Colony Grid | HR |  |  |
| Idaho Coordinate System * (see Note 2) | IA |  | TC |
| Illinois Coordinate System * (see Note 2) | IB |  | TC |
| Indiana Coordinate System * (see Note 2) | IC |  | TC |
| Indonesia Mercator Grid | ID |  | MC |
| Indonesia Polyhedric Grid * | IE |  |  |
| Iowa Coordinate System * (see Note 2) | IF |  | LE |
| Ivory Coast Azimuthal Grid | IG |  |  |
| Irish Cassini Grid | IH | EUR | CS |
| Ivory Coast Belt | IJ |  |  |
| Irish Transverse Mercator Grid | IK | IRL | TC |
| Iceland New Lambert Zone | IL | HJO | LE |
| India Zone * | IN |  | LE |
| Iberian Peninsula Zone | IP |  | LE |
| Iraq Zone * | IQ |  | LE |
| Iraq National Grid | IR |  | TC |
| Italy Zone * | IT |  | LE |
| Ivy - Found on an HA in Marshall Islands | IY |  |  |
| Iceland Zone | IZ | HJO | LE |
| Jamaica Foot Grid | JA |  | LE |
| Japan Plane-Rectangular Coordinate System | JB |  |  |
| Japan Gauss-Schreiber Grid | JC |  |  |
| Jamaica National Grid (metric) | JM |  | LE |
| Johore Grid | JO |  | CS |
| Austria Gauss-Krüger Grid | KA |  | TC |
| Bulgaria Gauss-Krüger Grid | KB |  | TC |
| Katanga Grid | KC |  |  |
| Kansas Coordinate System * (see Note 2) | KD |  | LE |
| Kentucky Coordinate System * (see Note 2) | KE |  | LE |
| Finland Gauss-Krüger Grid | KF |  | TC |
| German Gauss-Krüger Grid | KG |  | TC |
| Kenya Colony Grid | KH |  | CS |
| Korea Gauss-Schreiber Coordinate System | KJ |  |  |
| Louisiana Coordinate System * (see Note 2) | KK |  | LE |
| Lithuania Gauss-Krüger Grid | KL |  | TC |
| Kwantung Province Grid | KN |  |  |
| Turkey Gauss-Krüger Grid | KT |  | TC |
| Kwangsi Province Grid | KW |  |  |
| Luxembourg Gauss-Krüger Grid | KX | EUR | TC |
| Lambert Conformal Conic Grid * | LC |  |  |
| Latvia Coordinate System | LD |  |  |
| Levant Zone | LE | EUR | MJ |
| Levant Stereographic Grid | LF |  |  |
| Liberia Rectified Skew Orthomorphic Grid | LG |  | RS |
| Libya Zone | LI | EUR | LE |
| Lithuanian LKS-94 Grid | LK | EUT | TC |
| Sirte (Libya) Lambert Grid | LL |  | LE |
| Malaya Grid * | MA |  | CS |
| Malta Belt | MB | LOC | TC |
| Maldive-Chagos Belt | MC |  | TC |
| Madiera Zone | MD |  | LE |
| Mediterranean Zone * | ME |  | LE |
| Maine Coordinate System * (see Note 2) | MF |  | TC |
| Malaya Rectified Skew Orthomorphic (Yard) Grid | MG | KEA | RS |
| Martinique Gauss Grid | MH |  | TC |
| Maryland Coordinate System * (see Note 2) | MI |  | LE |
| Massachusetts Coordinate System * (see Note 2) | MJ |  | LE |
| Mexican Lambert Grid | MK |  | LE |
| Michigan Coordinate System * (see Notes 1 and 2) | ML |  |  |
| Mecca-Muscat Zone | MM |  | LE |
| Minnesota Coordinate System * (see Note 2) | MN |  | LE |
| Madagascar Grid (Laborde) | MO | TAN | LA |
| Mississippi Coordinate System * (see Note 2) | MP |  | TC |
| Morocco Zone * | MQ |  | MJ |
| Other Known Grid | MS |  |  |
| Missouri Coordinate System * (see Note 2) | MT |  | TC |
| Mauritius Zone | MU |  | LE |
| Montana Coordinate System * (see Note 2) | MV |  | LE |
| Mozambique Lambert Grid | MW |  | LE |
| Mozambique Polyconic Grid | MX |  | PH |
| Northwest Africa Zone | NA | MER | LE |
| New Jersey Coordinate System * (see Note 2) | NB |  | TC |
| Nigeria Colony Belt * | NC |  | TC |
| National Grid of Great Britain | ND | OGB | TC |
| Northern European Zone * | NE |  | LE |
| Nebraska Coordinate System * (see Note 2) | NF |  | LE |
| Numeric Grid | NG |  |  |
| New Hampshire Coordinate System * (see Note 2) | NH |  | TC |
| Niger Zone | NI |  | LE |
| Netherlands Stereographic Grid (Old Numbering) | NJ | PDM |  |
| North Korea Gauss-Krüger Grid | NK |  | TC |
| Netherlands Stereographic Grid (New Numbering) | NL | PDM |  |
| Netherlands East Indies Equatorial Zone British Metric Grid (Lambert) * | NM |  | MC |
| Nord de Guerre Zone * | NO |  | MJ |
| New Mexico Coordinate System * (see Note 2) | NN |  | TC |
| Nevada Coordinate System * (see Note 2) | NP |  | TC |
| New Sierra Leone Colony Grid * | NQ |  |  |
| New York Coordinate System * (see Notes 1 and 2) | NR |  |  |
| Netherlands East Indies Southern Zone | NS |  | LE |
| New Zealand Map Grid (NZMG) | NT | GEO | NT |
| Nicaragua Lambert Grid * | NU |  | LE |
| Niger Belt | NV |  | LE |
| North Carolina Coordinate System * (see Note 2) | NW |  | LE |
| North Dakota Coordinate System * (see Note 2) | NX |  | LE |
| Netherlands East Indies Equatorial Zone U.S. Yard Grid * | NY |  | LE |
| New Zealand Belt * | NZ |  | TC |
| Northern Malaya Grid | OA |  |  |
| Norway Gauss-Krüger Grid * | OB | OSL | TC |
| Ohio Coordinate System * (see Note 2) | OD |  | LE |
| Oklahoma Coordinate System * (see Note 2) | OE |  | LE |
| Orange Report Net | OR | NAS |  |
| Oregon Coordinate System * (see Note 2) | OS |  | LE |
| Palestine Belt * | PA |  | TC |
| Panama Lambert Grid | PB |  | LE |
| Palestine Civil Grid (Cassini) * | PC |  | CS |
| Paraguay Gauss-Krüger Grid | PD |  | TC |
| Peiping Coordinate System of 1954 | PE |  |  |
| Pennsylvania Coordinate System * (see Note 2) | PF |  | LE |
| Polish PSWG 1992 Grid | PG | EUT | TC |
| Peru Polyconic Grid | PI |  | PH |
| Philippine Plane Coordinate System | PJ | LUZ | PH |
| Poland Gauss-Krüger Grid | PK |  | TC |
| Poland Quasi-Stereographic Grid | PL |  |  |
| Philippine Polyconic Grid | PP | APL | PH |
| Portugal Bonne Grid, Old | PQ |  | BF |
| Portugal Bonne Grid, New | PR |  | BF |
| Portugal Gauss Grid | PS | LIS | TC |
| Puerto Rico & Virgin Islands Coordinate System * | PT |  | LE |
| Puerto Rico Lambert Grid | PU |  | LE |
| Qatar Cassini Grid | QA |  | CS |
| Qatar Peninsula Grid (or Qatar National Grid (TM)) | QU | QAT | TC |
| Russian Belt * | RB | EUR | TC |
| Reunion Gauss Grid | RC |  | TC |
| Rhode Island Coordinate System * (see Note 2) | RD |  | TC |
| Romania Bonne Grid | RE |  | BF |
| Soviet Coordinate System of 1942 * | RF | PUK | TC |
| Romania Lambert-Cholesky Grid | RH |  |  |
| Rikets National Grid * | RK | STO | TC |
| Romania Stereographic Grid | RI |  | SD |
| Pulkovo Coordinate System of 1932 | RT |  |  |
| South Africa Belt (yards) * | SA |  | TC |
| Senegal Gauss Conformal Grid (Belt) | SB |  | TC |
| South Africa Coordinate System (South Africa Belt (English feet)) * | SD |  | TC |
| Senegal Belt | SE |  | TC |
| South Carolina Coordinate System * (see Note 2) | SF |  | LE |
| Sahara Zone | SH |  | LE |
| South Dakota Coordinate System * (see Note 2) | SI |  | LE |
| South Libya Zone | SJ |  | LE |
| Sarawak Grid | SK |  | CS |
| Spain Lambert Grid | SL | EUR | LE |
| Southern New Guinea Grid * | SN |  | LE |
| South Georgia Lambert Grid | SQ |  | LE |
| South Syria Lambert Grid | SR |  | LE |
| Spanish North-Morocco Lambert Grid | SS |  | LE |
| Svalbard Gauss-Krüger Grid | SV |  | TC |
| Svobodny 1935 Coordinate System | SX |  |  |
| Seychelles Belt | SY |  | TC |
| Spitzbergen Zone | SZ |  | LE |
| Tanganyika Territorial Grid | TA |  |  |
| Tashkent 1875 Coordinate System | TB |  |  |
| Tennessee Coordinate System * (see Note 2) | TC |  | LE |
| Texas Coordinate System * (see Note 2) | TD |  | TC |
| Tobago Grid | TE | MDT | CS |
| Trinidad Grid | TF |  | CS |
| Trucial Coast Cassini Grid | TG |  | CS |
| Trucial Coast Transverse Mercator Grid | TH |  | TC |
| Turkey Bonne Grid | TI |  | BF |
| Tunisia Zone * | TN |  | MJ |
| Uganda Cassini Coordinate System * | UA |  | CS |
| Unidentified Grid | UB |  |  |
| Uruguay Gauss-Krüger Grid | UC |  | TC |
| Utah Coordinate System * (see Note 2) | UD |  | LE |
| Universal Polar Stereographic System * (Note: 61 is recommended Zone Number for Northern Polar Zone, -61 for Southern Polar Zone) | UP |  | PG |
| U.S. Polyconic Grid System | US | NAS | PH |
| Universal Transverse Mercator * (Note: 1 to 60 are recommended Zone Numbers for Northern Zones, -1 to -60 for Southern Zones) | UT |  | TC |
| Vermont Coordinate System * (see Note 2) | VA |  | TC |
| Virginia Coordinate System * (see Note 2) | VB |  | LE |
| Venezuela Modified Lambert Grid | VE |  |  |
| Vietnam Azimuthal Grid | VI |  |  |
| West Malaysia Rectified Skew Orthomorphic (Metric) Grid | WA |  | RS |
| Switzerland Bonne Grid | WB |  | BF |
| Switzerland Conformal Oblique Cylindrical Grid | WC |  | OC |
| West Virginia Coordinate System * | WD |  | LE |
| Wisconsin Coordinate System * | WE |  | LE |
| Wyoming Coordinate System * | WF |  | TC |
| Washington Coordinate System * (see Note 2) | WH |  | TC |
| World Polyconic System | WP |  | PH |
| Yugoslavia Gauss-Krüger Grid (Not Reduced) | YA | HER | TC |
| Yugoslavia Reduced Gauss-Krüger Grid | YG | HER | TC |
| Yunnan Province Grid | YU |  |  |

* grid category, covering more than one possible grid

Note 1. In this case, not all zones use the same projection.

Note 2. For US State Plane Coordinate Systems, the recommended grid zone number is the 4-figure code given by (a) FIPS PUB 70-1 for grids on North American Datum 1927 and (b) NOAA Manual NOS NGS 5 for grids on North American Datum 1983.